-*- text -*-

$Revision: 1.11 $

$Author: reida $

$Date: 2002/06/26 18:35:10 $

OOF2 Property Constructor's Guide

=================================

One of the major goals of the OOF2 project is to provide a

finite-element framework with "pluggable physics" -- constitutive

rules which govern the behavior of materials are modular, and

reasonably self-contained, so that users with sufficient coding

ability can write their own new properties, either in C++ or

in Python.

This document describes how to write a new property class for OOF2.

What Properties Must Do

-----------------------

From a computational point of view, the fundamental role of an OOF2

property is to provide a mechanism for relating fields -- the degrees

of freedom of the model system -- to fluxes, quantities whose

divergences correspond to generalized forces.  In OOF2, several

properties which contribute to the same flux are grouped into equation

objects, which manage the contributions from each of the properties,

and the set of equations is then used to construct the master

stiffness matrix.  The system is designed to be modular in such a way

that properties only need to know how to make their own contribution

to the appropriate flux.  Provided properties do this correctly, and

adhere to the API, the OOF2 system can handle all of the other levels

of abstraction, including the construction of equations, the

application of boundary conditions and so forth.

Property Registration Requirements

----------------------------------

In OOF2, a property object is an instance of a class which is a

subclass, either directly or through an inheritance chain, of the C++

"Property" class.  Properties written in Python must descend from the

PyPropertyWrapper class, which exposes the Property interface to

Python through SWIG, and is itself a subclass of Property.

There are a number of bookkeeping capabilities that property objects

must have, in order to behave correctly as members of a Material

object.  Each time a property is added to, or removed from, a

material, that material then iterates through all of its member

properties, calling all of their "registr" routines (so named because

the more obvious "register" is a reserved word in C++), and on

completion, looping through them all again calling their

"cross_reference" routines.

Properties must provide a "registr" function, and if they require the

cooperation of other properties, should extract pointers to those

properties in the "cross_reference" function.

The property "registr" function achieves its goal principally by

calling other, more "atomic" registration functions in other objects.

It takes as an argument the material undertaking the registration.

Properties indicate their type and the flux to which they contribute

by calling the argument's "registr" function with themselves and

either a string indicating their type, or a flux object to which they

contribute, as arguments.  The function indicates which fields are

required to compute the property in question require by calling that

field's "registr" function with a pointer to themselves as an object.

The property cross_reference function is called after all of a

material's property's registr functions have completed.  At this

point, the property can retrieve pointers to other properties by

calling the host material's fetchProperty routine with the appropriate

string argument.  If the corresponding material has registered itself

correctly, it will be retrieved by this means.  This mechanism should

be used, for instance, by non-elastic properties to get access to the

elastic constants.

These routines have to be supplied somewhere in the set of classes

above the actual property instance, somewhere below the Property base

class itself.  It is sensible to group properites with similar

registration requirements into class hierarchies headed by a class

which hosts a common "registr" function, providing the functionality

exactly once at the appropriate level.  For example, all elasticity

properties in the base version of OOF2 descend from an "Elasticity"

class, which has a registr routine which notes that elasticity

requires a displacement field, contributes to the stress flux, and

should be referred to by other properties as "Elasticity".

**EXAMPLE

For Python properties, the coding logistics are slightly different --

these properties must provide a list of fields and fluxes and a string

to the PyPropertyWrapper's Python init.  The PyPropertyWrapper stores

this data, and hosts a "registr" function which then uses the data to

inform the host material about required fields, contributed fluxes,

and the property type.

**EXAMPLE

Sequence of Events

------------------

Once a property is registered and cross-referenced, it is ready to be

called by the solver to provide numerical values for its contributions

to the fluxes.  The solver will call each of several entry points a

number of times as it iterates through the equations and elements

which make up the mesh.  All of these calls are present as virtual

functions in the base Property class, and so need not be provided by

every property unless it makes sense to do so.

The first property entry point called in the process of constructing

the master stiffness matrix is the precompute routine.  It gets called

from Material::precompute_all, which in turn is called from

Mesh::make_stiffness.  There are no arguments.  This call provides the

property with the opportunity to complete any high-overhead

computations which could not be completed earlier, but which are too

expensive to repeat element-by-element or node-by-node.  The canonical

example is for anisotropic properties to use this call to multiply

their coefficient matrices by their material's orientation matrix.

The material editor ensures that property instances are not shared

between materials, so you can be assured that this routine will only

be called once for a given equilibration.

Once the precomputations are complete, the main stiffness-matrix

construction loop begins iterating over elements in the mesh.  There

is at most one material per element.  For each element, the

corresponding material notifies each of its properties that an element

computation has begin by calling the property's begin_element()

routine with a pointer to the current element as an argument.  There

is a corresponding end_element() call, also with the element as an

argument, which is called when the element computation has been

completed.

The solver's next task is to decide how many integration points to use

in the element, for which it needs to know the polynomial order of the

expression being integrated.  This is handled in the material's

make_stiffness routine, which is called by Element::make_stiffness.

For each flux which is contributed to by properties in this material

(which is known from the registration data provided by the

properties), there is a list of properties which contribute.  For each

such property, a call is made to that property's K_integration_order

routine, with the polynomial order of the shape functions passed in as

an argument.  This function should return an integer equal to the sum

of the argument and the polynomial order of the computation being

made.  If the property's contribution to the flux is quadratic in the

fields, for example, and the element has shape functions of degree

two, the polynomial order of the resulting computation is four, and

this is what the K_integration_order routine should return.  The

material will provide enough integration points for the largest

polynomial order it encounters among the fluxes and properties.

At this point, the main property computation can begin.

Each flux in the material will, for each integration point in an

element, construct for itself a local matrix and a local vector to

which the property should make its contributions.  They are called the

"flux matrix" and the "flux offset".  The rows of the matrix are

indexed by the components of the equations which contain the flux, and

are eventually mapped to rows of the master stiffness matrix.  The

columns of the matrix are indexed by all the degrees of freedom in the

current element, which is to say, there is one column per component of

an active field per function-interpolation node.

The property makes its contribution to this flux matrix in its

fluxmatrix routine.  This routine takes as arguments a mesh, an

element, an ElementFuncNodeIterator object which indicates which node

in the element is current, a pointer to the flux, and an integration

point.  The property can use the ElementFuncNodeIterator to evaluate

the shape functions within the element at the indicated point by

calling the .shapefunction method with the current point as an

argument.  Derivatives of the shape functions are also available.  The

property should then iterate over the components of the flux to which

it contributes, and make contributions to the appropriate matrix

element, which can be retrieved (in reference form) by calling the

flux's matrix_element method with, in order, the mesh, the index of

the flux component, the field, the field component, and the

ElementFuncNodeIterator reference.  The flux will use the flux

component to find the row, and the field, field component, and

ElementFuncNodeIterator to determine the column.

A similar scheme is used if the property needs to make contributions

to the right-hand-side of the master stiffness matrix system.  The

property must provide a "fluxrhs" routine, which takes as arguments

the mesh, the element, the flux, and the integration point.  This is

useful for properties which make non-field-dependent contributions

directly to the values of the flux.  An example of this is thermal

expansion, where a temperature-dependent offset must be added to the

stress in order to provide for the correct stress-free strain.  Such

properties should make their contributions to the appropriate

"offset_element", retrieved from the flux by specifying the mesh and

the flux index to the offset_element method.

It may also happen that a property makes direct contributions not to

the flux, but to its divergence.  Such "body force" contributions also

occur in the fluxrhs routine, but in this case, rather than retrieving

the offset_element, the property should retrieve and write to the

"rhs_element" method, again specifying the index, and keeping in mind

that the dimensionality of this vector is that of the *divergence* of

the flux, not the flux itself.

After each element has been completed, and all the gauss points have

been passed in to fluxmatrix and fluxrhs, the property's "element_end"

entry point is called, with the element as an argument.  Properties

can clean up storage space used or intermediate results computed in

the process of making the field-to-flux mapping.

Note that the property itself does not do any of the integration.  It

need not (must not!) apply any weighting coefficients or do any other

processing other than computing the local contribution to the flux at

a given point.  Integration, along with matrix construction,

constraint resolution and the application of boundary conditions, is

the responsibility of the solver.  If you find you want to make

changes to these portions of the code, the right way to do that would

be to examine the solver code itself, rather than trying to sneak the

desired effects in via the property API.

Currently, the solution calls do not make any particular effort to

identify the location in space of a given integration point.  The

position passed to the fluxmatrix and fluxrhs routines is in the

element's master space.  Inhomogeneous properties which want to know

their location in the system should not use the element's shape

functions, because their host element may not be isoparametric, but

should instead call the element's from_master method, with the

integration point in master space as an argument, to find their

location in the system.

Properties written in Python have the same behavior and obey the same

API as described above.  The Python class representing a property

should descend from the PyPropertyWrapper class.  It must provide the

same functions, with the same arguments, as the C properties.

SWIG Shadow Objects for Properties

-----------------------------------

C++ properties have to be visible to Python, and so should be SWIG'd.

The constructor should be accessible, and the shadow class should have

a __repr__ function which returns a string which, when executed, calls

the registraiton object's __call__ method.  The __repr__ will, of

course, require access to the property's internal parameter values.

It is most sensible forproperties to also provide access to the

parameters separately, and to have the __repr__ function use these

parameter-extraction functions to build its return string.  There is

no formal requirement in the API for extraction of individual

parameters, but such a thing is not out of the question in future

versions.

PropertyRegistration Objects

----------------------------

In order to be operated correctly by the user interface, every

property imported into OOF2 must create a PropertyRegistration object

at import time.  The PropertyRegistration object will create actual

instances of the property at run-time by calling the constructor.  The

PropertyRegistration is a vital component of a Property object.  It

allows your property to accept parameter values either from the GUI or

from script or menu commands, and by inserting itself into the OOF

namespace under a name which you provide, controls the "public face"

of your property.  For C++ properties, this command must be in the

"_.py" file corresponding to the property module.

The PropertyRegistration object created at import-time should also be

assigned to a key in the property manager, which is available under

the name "AllPropreties".  By making this assignment, you place your

property in the property tree, making it visible to the GUI.  The key

is a string consisting of colon-separated names, the last of which

should be the same as the public name of your property.

The general format of this assignment is:

AllProperties['Head:Middle:Public_name'] =

PropertyRegistration('Public_name', ActualClassName,

'fully.qualified.module.name',

<ordering>, params=[...])

... where Public_name is the string presented in the tree, and the

name representing your property in the scripting namespace,

ActualClassName is the name of the class defined in the file,

fully.qualified.module.name is the dot-separated path to the file's

location in the source tree, starting from the first directory below

OOF2/SRC (this should be "engine"), <ordering> is some integer, and

the right side of the params assignment is a list of parameters your

property expects in its constructor, in the correct order.

OOF2 includes code templates for properties, both for Python and C++.

Users are encouraged to consult these templates.  Users may also find

it helpful to read the following section, which documents the

structure of the PropertyRegistration class.

Property Management and the PropertyRegistration Class

======================================================

Because of the special behavior of properties in OOF, their

registration objects are highly customized.  The departures from

default behavior revolve around the specialness of names in the

property class.  PropertyRegistration objects do not generally present

their "name" field as a parameter to the GUI, nor do they expect to

receive a name with parameter values passed in to the

"OOF.Property.Parametrize..." menu tree.  The name of a property

object is implicit in its location in the tree.  However, it is still

possible to create a named property (as from a script, for instance)

by calling an unnamed property's PropertyRegistration's __call__

method, which *can* take a name as a parameter.  If such an object

receives a name, it will not set its own name parameter, but will

instead create a NamedPropertyRegistration object from itself, and set

the new object's name parameter, and at the same time, automatically

add the object to the menu tree and the GUI, if present.

So, for example, given a starting point for which there is no instance

of the property "Verisimilitude" named "truth", both of the following

sets of operations will create one with the parameter "veracity" set

to 1.  The first would be what would appear in a log file as a result

of typical GUI operations to set up such a property.  The second would

be the result of saving the property to a file.

# Create via menu ops.

OOF.Property.Select(propname="Unphysical:Versimilitude")

OOF.Property.New(propname="truth") # Operates on current selection.

OOF.Property.Parametrize.Unphysical.Versimilitude.truth(veracity=1)

# Create by calling the PropertyRegistrations' __call__.

Verisimilitude(name="truth", veracity=1)

Additional discussion of the PropertyRegistration object and its

mechanics is contained in the materialeditor documentation.